Metadata: PTSP Summer 2008 Data.xls 


METADATA

Summer 2008 Dataset

Putney Study Plot (PTSP)

Senator Beck Basin Study Area (SBBSA), San Juan Mountains 

I.  Putney Study Plot (PTSP)

1) Site

a) Putney Study Plot (PTSP)

i) RF Station ID = 1

(1) 1 hour data arrays:  Flag field = 101

(2) 3 hour data arrays: Flag field = 103

(3) 24 hour data arrays: Flag field = 124

b) Site is private property used with the permission of landowner Tronox, LLC

c) Site unlocked; datalogger and battery enclosures locked with keyed padlock

d) Lat/Lon: 37( 53' 34" N, 107( 41' 42" W; NAD 1927

e) Elevation: 12,325 feet, 3,757 meters

f) Aspect: level 

g) Slope Angle: 0 degrees

h) Photographs: http://www.snowstudies.org/senbeck_sa.html
i) Changes to site: no alterations to tundra site since original 1970’s installation aside from removal of obsolete equipment from mast and cleanup of power and data land lines.
2) Operation Status 
a) Summer 2008 dataset from 0100 hrs June 21, 2008 (day-of-year 173) through 2400 hrs September 30, 2008 (day-of-year 274).

b) All data collected on Mountain Standard Time, by day-of-year (DOY)

i) System clocks not switched from/to Mountain Daylight Time in Fall or Spring

ii) 24 hour summary datasets (Flag = 224) are calendar day 

c) Measurements are calculated in both SI (International System of Units) and imperial/US units to facilitate use by other public safety agencies.

3) Type of Site

a) Study Plot: elevated and prominent 'sub-summit' location on N/S ridgeline.

4) Power

a) Solar power capacity upgraded in August 2007 from a 10 watt to a 20 watt photovoltaic panel providing regulated charge to two 90 amp hour gel 12V DC batteries.

5) Sensors

a) Wind Speed and Direction 

i) Properties

(a) Make: RM Young

(b) Model: #05103-5 

(c) Serial Number: WM61999

(d) Type: Wind Monitor

ii) Installation

(a) Height above ground: 9.5 m

(b) Distance from tower or obstacle: 0.5 m

(c) Date originally installed at PTSP: October 4, 2004  (DOY 278); 

(d) Service/calibration dates: new instrument at installation; original calibration July 9, 2004. 

(e) Comment: mounted to arm extending due north from mast

(f) Comment: Campbell Scientific Edlog software instruction P69, parameter 3 set at option 2 to calculate both ‘resultant’ and ‘scalar’ mean wind speed, calculate ‘resultant’ mean wind direction (only), standard deviation of mean wind direction.  Instruction P73 calculates peak wind gust.   

b) Air Temperature and Humidity

i) Properties

(a) Make: Campbell/Vaisala 

(b) Model: HMP-35C

(c) Serial Number: 7063

(d) Type: Vaisala humiter and Fenwall UUT51J1 thermistor

ii) Installation

(a) Height above ground: 3.1 m

(b) Date originally installed at PTSP: December 11, 2004 (DOY 346)

(c) Service/calibration dates: serviced and recalibrated by Campbell Scientific November, 2004.
(d) Comments: mounted on east side of mast at 3.1 m above ground to prevent damage from blowing gravel/rock/sand. 

6) Datalogger

a) Make: Campbell Scientific

b) Model: CR10X

c) Serial Number: 40362

d) Type: fully programmable measurement and control system with ring memory

e) Acquisition date: September, 2004

f) Date originally installed at PTSP: October 3, 2005 (DOY 277)

g) Service/calibration dates: original calibration certificate September 16, 2004

7) Data Retrieval

a) Radio telemetry using phone-to-RF base station; no repeater utilized

i) Campbell Scientific model RF 310M modem; serial #6928

ii) Model RF310 Maxon SD-125 V2 VHF radio: serial #030604688

iii) Cellwave PS1121-2 dipole antenna 

8) Software

a) Campbell Scientific LoggerNet 2.1c

b) One PTSP Edlog (LoggerNet 2.1c) programming version (PTSP0708V1) utilized during Summer 2008. 

9) Observer Contact Information

a) Name:  Chris Landry - Executive Director

b) Organization:  Center for Snow and Avalanche Studies

c) Address:  PO Box 190, Silverton, CO, USA 81433

d) Telephone:  (970) 387-5080

e) Email:  clandry@snowstudies.org
f) Website: http://www.snowstudies.org/index.html



10) Data processing and output methods (data points as labeled in Excel Spreadsheet):

a) Measurement execution interval:  5 seconds

b) Measurements: see table below 

Data processing and output methods (data points as labeled in Excel workbook PTSP Summer  2008 Data.xls):

	Spreadsheet Labels 

(some abbreviations expanded)
	Loc'n

#
	LoggerNet 2.1c

Ouput Instruction
	Type of

Measurement
	Notes

	Flag
	1
	P80
	na
	201 = 1 hour array

203 = 3 hour array

212 = solar noon array

224 = 24 hour array

	Year
	2
	P77
	Na
	Calendar year 

	Day of Year
	3
	P77
	na
	Sequential day of calendar year

	Time MST
	4
	P77
	na
	Mountain Standard Time at end of array period, in military time; previous day at midnight, 2400 at midnight

	Min Air C 

Time Minimum
	5

6
	P74
	Minimize
	Array minimum air temperature (C), time of minimum

	Max Air C

Time Maximum
	7

8
	P73
	Maximize
	Array maximum air temperature (C), time of maximum

	Air F Min
	9
	P74
	Minimize
	Array minimum air temperature (F)

	Air F Max
	10
	P73
	Maximize
	Array maximum air temperature (F)

	RH
	11
	P70
	Sample
	Samples relative humidity (%) once at end of array

	Peak Gust MPH 

Time Peak Gust

Peak Gust MS

S(Scalar) Avg Wd MPH

U(Resultant) Avg Wd MPH

Avg Wind Dir'n

Std Dev Dir'n

S(Scalar) Avg Wd MS

U(Resultant) Avg Wd MS

Avg Wind Dir'n

Std Dev Dir'n
	12

13

14

15

16

17

18

19

20

21

22
	P73

P69
	Maximize

Wind vector
	Array peak gust in MPH, MS

P69 output option '2': scalar mean horizontal wind speed S; resultant mean wind speed U; resultant (speed-weighted) mean wind direction; standard deviation wind direction using Campbell Scientific algorithm.

	Sys Volts
	23
	P70
	Sample
	Voltage at datalogger (prior to regulation) sampled once at end of array 

	Day Avg Air C
	24
	P71
	Average
	Averages 17,280 measurements of air temperature collected during a day, from 0000 hrs through 2400 hrs

	Day Avg Air F
	25
	P71
	Average
	Averages 17,280 measurements of air temperature collected during a day, from 0000 hrs through 2400 hrs


Page 3 of 3

